Contents

Arduino Programming Setup Guide	2

1. Install Arduino IDE	2

2. Setup Arduino Environment	2

3. Install Required Libraries	2

4. Load the Sketch	2

5. Upload the Code	2

Moving the arm (manual Serial Com Port)	3

Calibration of the starting point.	4

How to set the starting position of the arm(initialisation):	4

Creating a Pick and Place sequence using Serial Com port	5

Create a new movement.	5

1.	Program a movement stage: Move to Pick	5

1.	Program a movement stage:  Pick the Object	6

2.	Program a movement stage:  Move to Scan	6

3.	Program a movement stage:  Move to Place	7

4.	Program a movement stage:  Place Object	7

5.	Finish program and stop recording	8

6.	Play the PICK, SCAN and PLACE sequence	8

List of Commands in executeCommand	8

# Arduino Programming Setup Guide

## 1. Install Arduino IDE

- Download the Arduino IDE from the Arduino website.

- Choose the version compatible with your operating system (Windows/Mac/Linux).

- Follow the installation instructions provided on the website.

## 2. Setup Arduino Environment

- Connect your Arduino board to your computer using a USB cable.

- Open the Arduino IDE.

- Go to Tools > Board and select your Arduino board model.

- Go to Tools > Port and select the COM port that your Arduino is connected to.

## 3. Install Required Libraries

- Your code uses the Adafruit_PWMServoDriver library.

- In the Arduino IDE, go to Sketch > Include Library > Manage Libraries...

- In the Library Manager, search for “Adafruit_PWMServoDriver”.

- Find the library in the list and click “Install”.

## 4. Load the Sketch

- Open the 6_DOF_With_BT_04.ino file in Arduino IDE.

- Review the code and make any necessary modifications for your specific hardware setup.

## 5. Upload the Code

- With the Arduino still connected to your computer, click the “Upload” button in the Arduino IDE (right arrow icon).

- Wait for the message “Done uploading” in the status bar – this indicates the code is now running on your Arduino.

# Moving the arm (manual Serial Com Port)

The servos move in a range of 0-180 degrees. To find out the range of movement send the

commands tabled below:

- Open the Arduino IDE.

- Go to Tools > Serial monitor.

- Change the setting if necessary.

- Type the command into Cmd line box.

Note: You can choose and angle between 0-180.

| HIP 0 | Will move the servo fully to the 0-degree point |
| --- | --- |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

# Calibration of the starting point.

You may need to adjust the starting position of the arm, below are instruction for you to follow.

## How to set the starting position of the arm(initialisation):

- Open the 6_DOF_With_BT_04.ino file in Arduino IDE and find the function called:

- void MoveToStart() {

servoAngles[0] = 100; //HIP servo

servoAngles[1] = 90;  //WAIST servo

servoAngles[2] = 10;  //SHOUDLER servo

servoAngles[3] = 95;  //ELBOW servo

servoAngles[4] = 180; //WRIST servo

servoAngles[5] = 0;   //CLAW servo

- Make adjustments to the servo angle i.e., servoAngles[0] = 150; //HIP servo

- Upload the Code:
With the Arduino still connected to your computer, click “save” and then the “Upload” button in the Arduino IDE (right arrow icon).

- Wait for the message “Done uploading” in the status bar – this indicates the code is now running on your Arduino.

## How to set the speed of movement

Type command: speed 1-100

1 = full speed

100= slow speed

# Creating a Pick and Place sequence using Serial Com port

Open the Arduino IDE.

Go to Tools > Serial Monitor

Type the command into Cmd line box:

Command: MOVE_TO_START

Description: Commands the robotic arm to move to its starting or default position. This is used to initialize the position of the arm or to reset it.

## Create a new movement.

Command: START_RECORD

Description: Begins the recording of movements. This is used when you want to capture a new sequence of movements for later playback or analysis.

## Program a movement stage: Move to Pick

Command: See table:

Description: this is where you can choose the angle of movement you require to move the servo individually.

| Command: | Description |
| --- | --- |
| HIP 0 | Will move the servo fully to the 0-degree point |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

## Program a movement stage:  Pick the Object

Command: See table:

Description: this is where you can choose the angle of movement you require to move the servo individually.

| Command: | Description |
| --- | --- |
| HIP 0 | Will move the servo fully to the 0-degree point |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

## Program a movement stage:  Move to Scan

Command: See table:

Description: this is where you can choose the angle of movement you require to move the servo individually.

| Command: | Description |
| --- | --- |
| HIP 0 | Will move the servo fully to the 0-degree point |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

## Program a movement stage:  Move to Place

Command: See table:

Description: this is where you can choose the angle of movement you require to move the servo individually.

| Command: | Description |
| --- | --- |
| HIP 0 | Will move the servo fully to the 0-degree point |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

## Program a movement stage:  Place Object

Command: See table:

Description: this is where you can choose the angle of movement you require to move the servo individually.

| Command: | Description |
| --- | --- |
| HIP 0 | Will move the servo fully to the 0-degree point |
| HIP 90 | Will move the servo to the halfway point (90) |
| HIP 180 | Will move the servo fully to the 180-degree point |
| Repeat for WAIST, SHOUDLER, ELBOW, WRIST, CLAW | As above |

Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

## Finish program and stop recording.

Command: STOP_RECORD

Description: Stops the recording of movements. It is used to conclude the capturing of a movement sequence.

## Play the PICK, SCAN and PLACE sequence.

Command: PLAY_MOVEMENTS

Description: Triggers the playback of recorded movements. This command initiates a sequence of movements that have been previously recorded, for demonstrating or repeating a task.

# List of Commands in executeCommand

- Command: GETVALUE

Description: This command is used to retrieve the current status or value of a specific parameter. Used for debugging or verification.

- Command: MOVE_TO_START

Description: Commands the robotic arm to move to its starting or default position. This is used to initialize the position of the arm or to reset it.

- Command: START_RECORD

Description: Begins the recording of movements. This is used when you want to capture a new sequence of movements for later playback or analysis.

- Command: STOP_RECORD

Description: Stops the recording of movements. It is used to conclude the capturing of a movement sequence.

- Command: SAVE_MOVE

Description: Used to save a particular movement or a set of movements. This functionality allows the arm to learn and repeat specific actions.

- Command: PLAY_MOVEMENTS

Description: Triggers the playback of recorded movements. This command initiates a sequence of movements that have been previously recorded, for demonstrating or repeating a task.

- Command: HIP 0

Description: Will move the servo fully to the 0-degree point

- Command: WAIST 0

Description: Will move the servo fully to the 0-degree point

- Command: SHOULDER 0

Description: Will move the servo fully to the 0-degree point

- Command: ELBOW 0

Description: Will move the servo fully to the 0-degree point

- Command: WRIST 0

Description: Will move the servo fully to the 0-degree point

- Command: CLAW 0

Description: Will move the servo fully to the 0-degree point

Command: Speed 1

Description: Will set the speed to move the servo to top speed, Speed 50 will cause the movement to slow down.